Table of Contents

1	Goal	3

2	Current Materials	3

3	Overall System	3

4	Use Cosmic Watch to construct and test particle detector	3

5	Circuit Design	4

6	Radio Hardware and Protocol Design	7

6.1	APRS Radio System (1200bps)	7

6.2	Radio Modem (115200bps)	8

6.3	UAV Data Link (2.8Mbps)	8

6.4	SHARI	8

6.5	Conclusion	8

7	Particle Detector and Wireless Transmission Hardware Designs	8

7.1	Signal through ADC Pin	8

7.2	Signal through I2C Pins	9

7.3	Conclusion	9

7.4	Modification Plan	9

8	VHF Antenna	10

9	Power Supply Design	11

9.1	Lithium-ion Battery	11

9.2	Lithium Polymer Battery	11

9.3	Alkaline battery	11

9.4	Supercapacitor	11

9.5	Solar Panel	11

9.6	Lithium AA Batteries	11

9.7	Conclusion	12

9.8	Predicted Capacity	12

10	HAB collection and Recovery	14

10.1	Balloon + Parachute	14

10.2	Ascending Balloon + Parachute + Descending Balloon	15

10.3	Horizontal Adjustments	15

11	Design and Manufacture Encapsulation	16

11.1	Possible Designs	16

11.2	Conclusion	17

12	Test	17

12.1	Existing APRS System	17

12.2	Car-mount APRS System	17

13	System Mass	19

14	Launch	20

15	References	20

# Goal

Cosmic rays and muons produced in the upper atmosphere constantly rain down on the surface of the Earth. Here we propose to measure the flux (number of muons per unit area per second) as a function of altitude. The goal is to design a high-altitude air balloon systems that will be used to measure the flux of cosmic ray muons from the ground to an altitude of 20km. The HAB should be deployable, trackable, and recoverable.

Use silicon photomultipliers (SiPMs) to construct a low-mass detector that is simple, inexpensive, and robust. Data acquired by the detector and balloon tracking data will be transmitted to the ground and will be logged on board the balloon. The data thus acquired will be used to confirm the phenomenon of dilation predicted by Einstein’s Special Theory of Relativity and to test models of cosmic ray production in the stratosphere.

# Current Materials

- Purchased four plastic scintillators, which are expected to arrive in late February.

- Purchased SiPM and a set of CosmicWatch PCB and related parts. Although CosmicWatch might not be the most ideal payload, it can be used to test the scintillation detectors in the early stages.

# Overall System

Figure 3-1 Diagram of the system components

There is a general concept of the particle detector payload, but the specific designs are still being worked out as there are a few possibilities. The production and integration process of the HAB system is also being worked on.

# Use Cosmic Watch to construct and test particle detector

- Consider resources in http://www.cosmicwatch.lns.mit.edu/ and https://github.com/spenceraxani/CosmicWatch-Desktop-Muon-Detector-v2

- Analyze how Cosmic Watch is implemented, especially how the particle detector produces, collects, processes, and stores the signals.

- determine the tracking design.

# Circuit Design

P1-2 of the SiPM electrical circuit takes a 29.5V current, P3-4 is to ground, and P5-6 outputs to Cosmic Watch Main PCB.

Figure 5-1 Cosmic Watch Circuit Diagram

The SiPM DC-DC power supply circuit uses LT2461  to construct a DC-DC booster to boost the source 4.6V DC to a 29.5V for the SiPM electrical circuits.

Figure 5-2 Booster in SiPM

With the LT1807 dual operational amplifiers, signals collected by SiPM goes through amplification and peak detection. The signal is then sent to ADC port of the MCU to be converted into a digital signal to be displayed, stored, and outputted.

Figure 5-3 Main PCB of SiPM

The MCU Cosmic Watch uses is the Arduino Nano, and the peak signal is send to the ADC [0] pin.

Figure 5-4 Arduino Nano Circuit Diagram

Figure 5-5 Arduino Nano IO Diagram

Data collected by Cosmic Watch is then stored in the SD card on the microSD PCB. The PCB is connected to the main MCU with an 8-pin connector. The MCU uses SPI pin to read from and write to the SD card. The 3.3V stable current circuit constructed with AP2138N on the microSD PCB is used to power the SD card. HC4050 provides a buffer for the read, write, and clock from the MCU. Looking at the diagram, the design of this PCB is not logical. First, HC4050 takes a 4.6V current, which means that the max output of which is also 4.6V, exceeding the 3.3V limit of the SD card.

Figure 5-6 Circuit Diagram of the SD card reader

Also, since HC4050 is a one-directional buffer, D0 does not go through the buffer. This means that the buffer circuit does not improve the anti-EMI (anti-electromagnetic interference) capability of the SD card reader. It is suggested to replace this with a bidirectional buffer chip, such as the TXB0104 from Ti. This is a “4-Bit Bidirectional Voltage-Level Translator with Automatic Direction Sensing and ±15-kV ESD Protection.”

Figure 5-7 Digram of HC4050 vs Diagram of TXB0104

# Radio Hardware and Protocol Design

## APRS Radio System (1200bps)

Consider http://qrp-labs.com/lightaprs and https://github.com/lightaprs/LightAPRS-1.0

Software: Open Source

Weight: 9 grams

Dimensions： 3.5 cm x 6 cm

IDE: Arduino

Platform: MightyCore

CPU: Atmega1284P-AU

Flash: 128 kB

Ram: 16 kB

EEPROM: 4 kB

Operating Frequency: 8 MHz

Operating Voltage: 3.3 Volt

Input Voltage: 4.5 (min) - 10 (max) Volt via usb or VBat pin

BOD: 2.7 Volt

Sensor: BMP180 (pressure and temperature)

VHF Radio Module: Dorji DRA818V (included)

Figure 6-1 APRS Board

VHF Radio Operating Frequency: 144-146 Mhz (configurable by code)

VHF Low Pass Filter: Available (7 elements)

VHF Radio Power: 0.5 Watt or 1 Watt (configurable by code)

VHF Power Consumption (TX): ~460 mA (0.5 Watt) / ~760 mA (1 Watt) (Automatically selected based on input voltage by code)

HF Radio Module: Si5351A-B-GT (included)

HF Radio Operating Frequency: 2.5kHz - 200Mhz (configurable by code)

HF Low Pass Filter: No

## Radio Modem (115200bps)

Models like GEMDS EL-806 have a theoretical transmission distance of 15km, but the actual distance is unknown (GE MDS).

## UAV Data Link (2.8Mbps)

These devices, such as DJI Lightbridge 2, have a short transmission distance of 5km, which is not suitable (DJI).

## SHARI

SHARI(SR-FRS1W Ham Allstar Radio Interface) is designed for construction and uses in the amateur radio service, works with a Pi2/Pi3 or Pi4, SHARI RF output power is 500 to 1000(default) milliwatts.

The UHF nodes are $5 less expensive than the VHF nodes.  Since Pi3B+ are slightly less expensive than Pi4Bs, I think our least expensive node using a SHARI would be a SHARI Pi3U (UHF, plugs into a Pi3).

A SHARI Pi3U kit is $60.  We can build the kit for you in which case the cost is $85.  You must provide the Raspberry Pi, a Pi case, a Pi power supply and a 4 GB or greater microSD card.

If you have a radio with DIN connector packet interface, you can use a DINAH which is $35 for the kit and $45 assembled.  You still supply the Pi and accessories.

- Designed for RPi 2, 3, or 4 so it might not be most suitable

- Does not come with CPU, GPS, thermometer, barometer.

- Not as integrated as LightAPRS

## Conclusion

Taking reliability, transmission rate, weight, and cost into consideration, Radio APRS system is the most ideal design for this project.

# Particle Detector and Wireless Transmission Hardware Designs

## Signal through ADC Pin

The peak signal detected by the particle detector is directly sent to the wireless transmitter and then processed by an ADC converter to collect, store, and transmit the signal. This requires the wireless transmitter to have an ADC pin and the detector only needs to send an analog signal, and MCU is necessary. While the LightAPRS MCU chip ATmega1284 has 8 ADC pins (PF0-7), none was drawn out by the board.

## Signal through I2C Pins

Since on LightAPRS, only the I2C and SPI pins have pads, the detector needs the MCU to collect the analog signal and send it through the I2C pins.

## Conclusion

To simplify the circuit and reduce both weight and power consumption, redesigning the Front Electronics board to simplify (remove redundant functions) and connect directly to the APRS board is ideal.

## Modification Plan

Since LightAPRS does not provide ADC breakout, we can use the RST pad. By cutting off the connection of the pad to the CPU Atmega1284, we can reconnect pin 37 (ADC0) from the Atmega1284 to the pad and keep SCK , MISO, MOSI pins for SPI to connect  microSD reader.

Figure 7-1 planned modifications to the APRS board

Then, the front PCB of the particle detector will also need to be redesigned.

Figure 7-2 Electrical Diagram of the new front board

# VHF Antenna

According to FCC, frequency of APRS is 144.39Mhz. For simplicity’s sake, quarter wave monopole antenna is the best choice.

lantenna=14×299792458ms144.39MHz=14×2.0763=0.51907m=51.907cm

Thus, taking a copper wire of length 51.91cm and solder it onto the middle pad of A1.

Figure 8-1 Position of the antenna

# Power Supply Design

## Lithium-ion Battery

Have High energy density and multiple configurations of voltage, but functions poorly in cold temperatures and charges slowly.

## Lithium Polymer Battery

Since li-po uses a solid, gel-like electrolyte, it is more robust and can withstand harsher weather conditions. However, it is less energy dense and more costly.

## Alkaline battery

Has lower energy density than lithium batteries and is single use. Has even worse low-temperature performance.

## Supercapacitor

Good low-temperature performance and changes quickly, but is large

## Solar Panel

Accessible energy than can be collected while HAB is in the air but cannot provide stable current and is prone to changes in weather.

## Lithium AA Batteries

- Energizer Ultimate Lithium AA batteries are the world's longest-lasting AA batteries

- Superior performance in extreme temperatures from -40 degrees F to 140 degrees F ensure reliable use in all seasons

- These double A batteries are leakproof batteries, guaranteed based on standard use

- AA battery 24 pack lasts up to 20 years in storage

- Use these double A batteries in high-tech items, like digital cameras or game controllers, and household items such as smoke detectors, toys, and games

The battery has a 1.5v constant discharge in 3000mAh lithium cell. This means for a 4.5V 6000mAh consumption, 6 pieces are needed.

Figure 9-1 18650 Rechargeable Lithium Ion Battery

## Conclusion

Lithium-ion is the more ideal design. The capacity needs to be determined by the consumption of the detectors and APRS, the transmission frequency (per second or per minute) and the time it takes for the HAB to rise to the designated altitude (15-20km).

## Predicted Capacity

PSiPM=29.5V×20mA=590mW

PLT1807=2×5V×13mA=130mW

The standby current for the SD card is 950μA and the average operation current is 200mA. Assume that SD card is on 50% of the time

PMicroSD=12×3.3V×200mA+950μA=332mW

The Power consumption of the VHF is 760mA 1W. Assume that it is on 20% of the time (twice per minute)

PVHF=15×5V×760mA=502mW

PATmega1284=5V×11mA=55mW

PUblox MAX-M8Q=5V×23mA=115mW

PCH340=5V×20mA=100mW

POther=5V×5mA=25mW

PLightAPRS=502mW+55mW+115mW+100mW+25mW=797mW

PTotal=590mW+130mW+332mW+797mW=1849mW

Take the voltage conversion loss into consideration, and assume the efficiency to be 80%, the total power needed is

PBattery=1849mW80%=2.3W

With http://tools.highaltitudescience.com/ I can calculate the time it takes for the HAB to rise. Assume the flight duration to be 3 hours, and double the amount of extra, taking into consideration of time to find payload once landed and the efficiency of the battery, the entire trip will be 9 hours. Costing a total of

EBattery=2.3W×9h=20.7Wh

If we use 5V battery packs, the capacity will need to be

Q5V=20.7Wh5V=4.14Ah=4140mAh

If we use 3.6V battery packs, the capacity will be

Q3.3V=20.7Wh3.6V=5.75Ah=5750mAh

# HAB collection and Recovery

## Balloon + Parachute

Figure 10-1 Design of the parachute

## Ascending Balloon + Parachute + Descending Balloon

Figure 10-2 After the carrier balloon pops, the parachute balloon will decrease the downward acceleration while the payload falls

## Horizontal Adjustments

Adding a propeller and a servo (used to change direction of propeller) to the base of the HAB. To connect for orientation, would it also be logical to add an electrical compass?

Figure 10-3 Potential Propeller Design

# Design and Manufacture Encapsulation

Due to the low environmental temperature of the system (-55°C), the lowest working temperature of each component of the system (including batteries) must be considered. The chips only function >40°C and the battery only functions >0°C. Thus, thermal insulation and heating is needed.

Figure 11-1 Temperature of the environment as altitude increases

## Possible Designs

- Insulation Material

- Electric heating + insulation

- Chemical heating + insulation

## Conclusion

After consulting some references, just the insulation material should be enough keeping the temperature relatively high. The material chosen is polystyrene foam.

# Test

## Existing APRS System

Find local APRS Repeater location or contact local HAM clubs and use their stations to complete the APRS signal receiving, transferring, and uploading to the server.  All I need to do is to visit the website to check for the data.

This requires little setup, no extra devices and little cost, but it also heavily depends on outside factors and makes the testing difficult.

## Car-mount APRS System

Configure my own system, including VHF Receiver, antenna, and APRS decoder (software or hardware).

This makes ground testing easy as I can configure this to be car-mounted (mounting brackets, source power from car, car-mounted antenna). This also allows me to track the flight path of the HAB in real-time, which makes recovery easier. However, this does have a cost associated with it and requires setup.

- Radio: consider Yaesu FT-2800M 2m 144MHz FM Mobile Transceiver Radio

- Receiver: Realistic Pro-2022

| Type: | VHF/UHF receiver/scanner |
| --- | --- |
| Frequency range: | 68-88 / 108-174 / 380-512 / 806-960 MHz (Europe) 30-54 / 108-136 / 138-174 / 380-512 / 806-960 MHz (USA) |
| Mode: | AM/FM |
| Receiver system: |  |
| Sensitivity: |  |
| Selectivity: |  |
| Image rejection: |  |
| Voltage: | 13.8 VDC or mains |
| Current drain: | Max ? mA |
| Impedance: | 50 ohms / BNC or telescopic whip |
| Dimensions (W*H*D): | ? mm |
| Weight: | ? Kg |
| Manufactured: | Japan, 19xx-19xx (Discontinued) |
| Other: | 200 memories |
| Related documents: |  |
| Modifications: |  |

- APRS Hardware Decoder: APRS Data Link + Data Cable for Yaesu

- APRS Software Decoder: Direwolf

Dire Wolf is a software “soundcard” AX.25 packet modem/TNC and APRS encoder/decoder. It can be used stand-alone to observe APRS traffic, as a tracker, digipeater, APRStt gateway, or Internet Gateway (IGate) . In the early days of Amateur Packet Radio, it was necessary to use an expensive “Terminal Node Controller” (TNC) with specialized hardware. Those days are gone. You can now get better results at lower cost by connecting your radio to the “soundcard” interface of a computer and using software to decode the signals.

This can be used in both ground testing and in the actual flight. We can use Radio + PC + SW to independently receive and save APRS data to decode the GPS data and track the HAB.

# System Mass

Plastic scintillator:

Vsci=5cm×5cm×1cm

ρsci=1.032gcm3

msci=5cm×5cm×1cm×1.032gcm3=25.8g

Outer cylindrical shell:

Δr=0.3cm

r=5cm

h=15cm

Vshell=2π×5cm×15cm-2×0.3cm×0.3cm+2π×5cm+0.3cm22×0.3cm=186cm3

ρsci=1.43gcm3

msci=186cm3×1.43gcm3=266g

| Component | Mass |
| --- | --- |
| Plastic scintillator | 25.8g |
| SiPM PCB | 10g |
| Particle Detector Front Board | 10g |
| LightAPRS | 9g |
| Antenna | 5g |
| microSD PCB | 5g |
| Battery (3.6V/6000mAh) | 100g |
| Insulation | 20g |
| Shell Latex balloon | 266g |
| Total: | 446g |

# Launch

Balloon selection should consult Understanding High-Altitude Balloon Flight Fundamentals and launch location, weather, and landing prediction should consult High Altitude Balloon Flight Predictions (Voss, Ramm and Dailey) (Stafford). Tool written for NOAA balloon flight predictions is available from: https://www.ready.noaa.gov/READY_balloon.php

# References

DJI. "DJI Lightbridge 2 User Manual V1.6." 20 Feburary 2017. DJI. PDF File. 20 December 2022. <https://dl.djicdn.com/downloads/lightbridge2/20170217/DJI+Lightbridge+2+User+Manual+V1.6.pdf>.

GE MDS. "GE MDS TransNET OEM EL806 Manuals." June 2007. manualslib. PDF File. 25 December 2022. <https://www.manualslib.com/products/Ge-Mds-Transnet-Oem-El806-9236134.html>.

Stafford, Sara. "High Altitude Balloon Flight Predictions." n.d. PDF File.

Voss, D. Voss, Natalie A. Ramm and Jeff Dailey. "Understanding High-Altitude Balloon Flight Fundamentals." Academic High-Altitude Conference. Indiana, 2012. 74-83. PDF File.